The User's Guide to Scheduling on the common border between SEPS and Ukrenergo

Trader's Manual Version 3.0

Effective from the trading day of November 12, 2026.

TABLE OF CONTENTS

1	INTRODUCTION	4

1.1	Purpose of this Document	4

1.2	Scope of this Document	4

1.3	Definitions and abbreviations	4

2	General Definitions	6

2.1	Business processes	6

2.2	Communication to TSOs and AO	6

2.3	Description of processes	6

2.3.1	Timeline	6

2.3.1.1	Long term timeframe	6

2.3.1.2	Day-ahead timeframe	7

2.3.1.3	Intraday timeframe	7

2.4	Cross border Nomination	8

2.4.1	Example of cross nominations	9

2.4.2	TSOs´ reactions	10

2.4.3	Long term Nomination	12

2.4.3.1	Nomination before availability of the Rights Document	12

2.4.3.2	Nomination after reception of the Rights Document	12

2.4.4	Daily Nomination	13

2.4.5	Intraday nomination process	13

2.5	Matching process	14

2.5.1	Correction Cycle, Matching at COT	14

2.6	Curtailment of nominations	15

3	Appendix	16

3.1	ETSO ESS 2.3/ESS 3.3	16

3.1.1	Definitions	16

3.1.2	Example	17

3.2	Referenced documents	19

# INTRODUCTION

## Purpose of this Document

The User’s Guide for Scheduling provides a description of all scheduling processes and the requirements of the scheduling IT systems to be implemented by electricity trading companies at the common border between SEPS and Ukrenergo (participating TSOs).

## Scope of this Document

This document was developed by the representatives of the participating TSOs using state-of-the-art technology and best business practices in the field of cross-border schedule nomination.

For the sake of clarity, the document provides a general description of the functionalities of all scheduling processes; however, its main objective is to precisely define the necessary requirements for the Users´ IT systems, as well as to describe possible responses or scenarios in the event of a nomination mismatch.

National requirements, internal matters of individual Traders, and rules applicable to borders with other regions, are not covered by this document.

## Definitions and abbreviations

| Abbreviation | Description | Remark |
| --- | --- | --- |
| AO | Allocation Office | Currently Joint Allocation Office S.A. (JAO) |
| BRP | Balance Responsible Party | A Balance Responsible Party is responsible for its imbalances, meaning the difference between the energy volume physically injected to or withdrawn from the system and the final nominated energy volume, including any imbalance adjustment within a given imbalance settlement period. |
| CAI | Capacity Agreement Identification |  |
| CCT | Capacity Contract Type |  |
| COT | Cut off time |  |
| fCNF | Final CNF | Final CNF sent to the BRP/ITR. |
| GCT | Gate closure time |  |
| iCNF | Intermediate CNF | Intermediate CNF sent to the BRP/ITR. |
| ITR | Interconnection Trade Responsible | Balance Responsible Party which is known by the TSO as the entity entitled to use the capacity rights |
| JAO | Joint Allocation Office S.A. | An allocation service company that hosts Europe's platform for cross-border transmission capacity. On behalf of TSOs, JAO auctions the available long and short-term transmission capacity rights. |
| LMR | Local market rules |  |
| PTR | Physical transmission right | PTRs entitle the rights holder to nominate parts or all of its PTRs at the TSO for physical delivery beyond the particular border. |
| RC | Reason code |  |
| RD | Rights document |  |
| TS | Time series |  |
| TSO, SO | Transmission System Operator |  |

# General Definitions

## Business processes

The business process between traders and TSOs is standardized, particularly with regard to timings and communication.

Scheduling is carried out seven days a week, regardless of local public holidays in the relevant areas.

## Communication to TSOs and AO

The defined communication standards are

- ETSO Scheduling System (ESS) 2.3/3.3 – according to local market rules

- ETSO Capacity Allocation and Nomination System (ECAN) 4.0

- ENTSO-E General Code list For Data Interchange

- ENTSO-E Acknowledgement Document (EAD) 5.0

- ENTSO-E EIC Reference Manual 5.3

## Description of processes

### Timeline

One of the preconditions for the smooth operation of the scheduling process is the harmonization of the timeline. The table below describes the main activities relevant to scheduling. The listed timeline was agreed upon by the participating TSOs. The timeline of the scheduling process is specified as follows:

#### Long term timeframe

Long term physical transmission rights (LT PTRs) products shall be allocated on UA/SK border via the course of explicit bilateral auctions performed by JAO.

The following timings shall be applicable:

| Process | Border | Start of the process (CET) | End of the process (CET) |
| --- | --- | --- | --- |
| Receiving of RD after Secondary Market | SK-UA | D-2 12:30 | D-2 12:45 |
| LT PTRs nomination | SK-UA | D-60 (SEPS) | D-1 8:00 |
| Correction cycle LT PTRs | SK-UA | D-1 8:00 | D-1 8:30 |
| LT PTRs matching cycle at COT | SK-UA | D-1 8:30 | D-1 8:45 |
| Curtailment |  |  |  |  |
| In case of curtailment after LT GCT: TSOs applies the reduction factor to the nominated Long term PTRs | After LT matching is finalized |  | asap (no dedicated timeslot foreseen) |
| In case of curtailment after LT GCT: Deadline for curtailment of LT nominations |  |  | D-1 9:00 |
| In case of curtailment after LT GCT: Matching after curtailment of LT nominations |  |  | asap (no dedicated timeslot foreseen) |

#### Day-ahead timeframe

Daily physical transmission rights shall be allocated on the UA/SK border via the course of explicit bilateral auctions performed by JAO.

The following timings shall be applicable:

| Process | Border | Start of the process (CET) | End of the process (CET) |
| --- | --- | --- | --- |
| Receiving of RD with PTRs allocated within Daily Auction procedure | SK-UA | D-1 10:50 | D-1 10:55 |
| Daily nomination | SK-UA | D-1 10:55 | D-1 14:30 |
| Correction cycle Daily | SK-UA | D-1 14:30 | D-1 15:30 |
| Daily matching cycle at COT | SK-UA | D-1 15:30 | D-1 15:45 |
| Curtailment |  |  |  |  |
| In case of curtailment: TSOs applies the reduction factor to the nominated LT and Daily PTRs | After day-ahead matching is finalized |  | asap (no dedicated timeslot foreseen) |
| In case of curtailment: Deadline for curtailment of LT and ST nominations after Daily GCT |  |  | D-1 16:20 |
| In case of curtailment: Matching after curtailment of LT and Daily nominations after Daily GCT |  |  | asap (no dedicated timeslot foreseen) |

Parties reserve the right to amend the timing of the processes and are obliged to inform about any changes to market participants in case of such amendment.

#### Intraday timeframe

Intraday physical transmission rights (ID PTRs) products shall be allocated on the UA/SK border through explicit bilateral auctions performed by JAO, namely the two intraday auctions ID1 and ID2.

In the first auction (ID1), ID PTRs for each hour of delivery day D (from 00:00 to 24:00) shall be allocated.

In the second auction (ID2), ID PTRs for the last 12 hours of delivery day D (from 12:00 to 24:00) shall be allocated.

The following timings shall be applicable:

| Process | Border | ID section | Start of the process (CET) | End of the process (CET) |
| --- | --- | --- | --- | --- |
| Receiving of RD with PTRs allocated within Intraday Auction procedure | SK-UA | ID1 | D-1 17:10 | D-1 17:11 |
|  |  | ID2 | D 09:11 | D 09:12 |
| ID nomination | SK-UA | ID1 | D-1 17:11 | D-1 22:00 |
|  |  | ID2 | D 9:12 | D 10:00 |
| Correction cycle ID | SK-UA | ID1 | D-1 22:00 | D-1 22:30 |
|  |  | ID2 | D 10:00 | D 10:30 |
| ID matching cycle at COT | SK-UA | ID1 | D-1 22:30 | D-1 22:45 |
|  |  | ID2 | D 10:30 | D 10:45 |
| Curtailment |  |  |  |  |
| In case of curtailment: TSOs applies the reduction factor to the nominated LT, ST and ID PTRs |  |  |  | asap (no dedicated timeslot foreseen) |
| In case of curtailment: Deadline for curtailment of LT, ST and ID nominations after Intraday GCT |  |  |  | asap (no dedicated timeslot foreseen) |
| In case of curtailment: Matching after curtailment of LT, ST and ID nominations after Intraday GCT |  |  |  | asap (no dedicated timeslot foreseen) |

## Cross border Nomination

When a document with a schedule nomination is received by the TSO, it is immediately subject to a formal check. If the result of the formal check is positive, the ITR receives an Acknowledgement message (ACK) with the reason code A01. In case of formal errors, the TSO doesn’t accept the document. Reason codes (RCs) and reason texts are provided in the ACK.

Nomination without RD: This type of nomination is subject to local market rules.

If validation against the Rights Document cannot be performed during the formal check due to the unavailability of the Capacity Rights Document, the ITR will be informed via an additional reason code A75 in the ACK report.

Nomination with RD:

If the Capacity Rights Document is available to the TSO at the time of receiving the nomination, the ITR will be informed of any exceeded capacity rights via an ANO report.

If the Capacity Rights Document is received later, or if a nomination from another ITR leads to an exceedance of capacity rights, the ITR will be informed via an ANO report once the issue is detected.

A nomination will be accepted under the following conditions:

- The sender (ITR) must have grid access in the area of the receiving TSO.

- The sender must be responsible for the party in the area of the receiving TSO.

Any error regarding these conditions will result in a negative ACK.

A CAI and CCT of a nomination are valid if:

- The combination of CCT and CAI exists in the RD

- At least one of the InParty or OutParty in the nomination is the RightsHolder specified in the RD.

Any error regarding these conditions results in an Anomaly or negative ACK, depending on LMR.

The capacity rights are provided in hourly resolution. Local market rules may require 15-minute resolution for nominations. In such cases, the ITR must send four identical values for each quarter-hour within one hour. TSOs must verify the presence of four identical values for each quarter-hour within the same hour. Violation of this condition will result in the rejection of the nomination.

Cross nominations (N:M) are allowed. That means that the InParty and Outparty do not need to be the same. Moreover, the ITR holding the capacity rights may use the allocated rights with more than one counterparty on the opposite side of the border.

### Example of cross nominations

TSO1 (EIC_TSO_1)

TSO2 (EIC_TSO_2)

Capacity right

CAI: T1T20001

CCT: A03 (right from the monthly auction)

Owner: ITR1

ITR1 (EIC_ITR_1)

ITR3 (EIC_ITR_3)

ITR4 (EIC_ITR_4)

ITR2 (EIC_ITR_2)

ITR1 (EIC_ITR_1)

Basic Rules:

- The direction of the nomination must match the direction of the corresponding capacity right.

- One of the ITRs on either side must be the owner of the capacity right. In the case of a 1:1 nomination (a special case of cross-border nomination), the ITRs on both sides and the owner are the same entity

- The total volume in the nominated Time series using the same CAI must not exceed the volume of the respective capacity right

Headers of nomination schedules of ITR1 to TSO1:

<InArea codingScheme="A01" v="EIC_TSO_2"/>

<OutArea codingScheme="A01" v="EIC_TSO_1"/>

<InParty codingScheme="A01" v="EIC_ITR_2"/>

<OutParty codingScheme="A01" v="EIC_ITR_1"/>

<CapacityContractType v="A03"/>

<CapacityAgreementIdentification v="T1T20001"/>

<InArea codingScheme="A01" v="EIC_TSO_2"/>

<OutArea codingScheme="A01" v="EIC_TSO_1"/>

<InParty codingScheme="A01" v="EIC_ITR_1"/>

<OutParty codingScheme="A01" v="EIC_ITR_1"/>

<CapacityContractType v="A03"/>

<CapacityAgreementIdentification v="T1T20001"/>

<InArea codingScheme="A01" v="EIC_TSO_2"/>

<OutArea codingScheme="A01" v="EIC_TSO_1"/>

<InParty codingScheme="A01" v="EIC_ITR_4"/>

<OutParty codingScheme="A01" v="EIC_ITR_1"/>

<CapacityContractType v="A03"/>

<CapacityAgreementIdentification v="T1T20001"/>

Header of nomination schedule of ITR3 to TSO1:

<InArea codingScheme="A01" v="EIC_TSO_2"/>

<OutArea codingScheme="A01" v="EIC_TSO_1"/>

<InParty codingScheme="A01" v="EIC_ITR_1"/>

<OutParty codingScheme="A01" v="EIC_ITR_3"/>

<CapacityContractType v="A03"/>

<CapacityAgreementIdentification v="T1T20001"/>

Nomination schedules to TSO 2 will be generated similarly.

### TSOs´ reactions

The result of a validation check will either lead to the rejection of the nomination or to the provision of additional information in an ANO report (depending on local market rules). This applies in the following use cases:

- A non-existent or incorrect CAI or CCT for the given InArea/OutArea combination,

- The ITR is not listed as either the InParty or OutParty in the nomination

- Schedules within one document exceed the capacity rights associated with the corresponding CAI

- a new ITR schedule is received and the total of all nominations using the same CAI exceeds the capacity right. In this case, an ANO report must be sent to all ITRs whose nominations refer to this CAI.

- a new RD is received and the total of all nominations using the same CAI exceeds a reduced capacity right. An ANO report must be sent to all ITRs whose nominations refer to this CAI.

An ANO report will always include the original values submitted by the sender and, in case of mismatch after GCT, also the counterpart’s Time series.

In the event of nomination rejection, reason codes and reason texts are provided in the Acknowledgement message.

In addition to capacity right exceedances, the ANO report may also include detected mismatches, depending on the process step.

Table of ACK/Anomaly Report Reason Codes (TimeSeries Level):

| RC | Reason Text | Document Type | ITRs Action |
| --- | --- | --- | --- |
| A62 | BusinessType is not “A03” | ACK (rejection) | Correct the BusinessType to “A03” and send again |
| A76 | BusinessType is “A03” but CAI and/or CCT doesn’t exist in the schedule-header | ACK (rejection) | Add CCT and/or CAI and send again |
| A59 | Quarterly-hour values in one hour not equal | ACK (rejection) | Correct the values so that all 4 quarterly-hour values of one hour are equal. |
| A22 | Invalid Party at the own side | ACK (rejection) | Correct own EIC and send it again |
| A57 | Deadline limit exceeded | ACK (rejection) | The document contains changed nominations and/or added nominations for which the deadline was exceeded. Correct to the last confirmed nominated values and send the document again if necessary |
| A76 | CAI inconsistency | ANO *) or ACK (rejection) | Correct the nomination in accordance with the RD and local market rules and send it once again. If the Time series is a zero Time series no ANO will be sent. |
| A22 | InParty or OutParty must be rights holder (ITR) | ANO *) or ACK (rejection) | Correct InParty or OutParty or delete the Time series from the document or set the values of the wrong Time series to zero. The precise proceeding depends on local market rules |
| A27 | Cross Border Capacity exceeded | ANO **) or ACK (rejection) | Correct Time series or contact rights owner, error can be a result of other traders´ nomination |
| A28 | Counterpart Time series missing | ANO *) | Contact counterpart in other area and correct Time series |
| A22 | Invalid Party on counterpart’s side | ANO *) | Correct nomination and send it again |
| A09 | Counterpart Time series quantity differences | ANO **) | Contact counterpart in other area or correct values in Time series |
| A63 | TS modified. | ACK rejection | Correct to the last confirmed nominated values and send the document again if necessary. |
| A09 | Time series quantity differences in one of the TS using the same CAI | ANO **) | Contact ITR and correct values in Time series |

*) Anomaly Report after SO-SO matching contains the original Time series of the ITR.

**) Anomaly Report after SO-SO matching contains both the original Time series of the ITR and the counterpart’s original Time series in case of mismatches. If the capacity limit is exceeded, only the ITR’s own Time series will be transmitted.

Please note that the rules concerning global (scheduling) positions depend on the local market rules. This means that in some market areas, the corrected nomination requires an ITR to skip Time series with invalid CAI+CCT, while in other areas, the ITR must provide the Time series with the invalid header information and set the values to zero.

All these issues can be resolved bilaterally between the ITRs, and it is not necessary to contact the TSO.

### Long term Nomination

#### Nomination before availability of the Rights Document

Long term nominations may be submitted/modified before the RD from the AO has been received, see 2.3.1.1.

The nominations will be checked for formal correctness. If the nominations are formally correct, a positive ACK-report will be sent, otherwise, a negative ACK-report will be sent, and the ITR can retry the nomination after making the necessary corrections.

#### Nomination after reception of the Rights Document

The validation of nominations starts once the secondary market is finished and the RD has been successfully received by TSOs. This process is triggered whenever:

- A new RD is received

- A new nomination is received

Validation according to article  2.4.2 TSOs´reactions is performed.

### Daily Nomination

After the publication of daily auction results nominations of daily rights can be sent or modified. During this period, the ITR may submit schedules. According to local market rules and the stage of the process, the TSO may send:

- confirmation that the schedule was received

- Positive or negative Acknowledgement Document

- Anomaly Report

In case of errors, the ITR should correct the nominations as soon as possible.

Daily nomination process respects the same rules as the Long-term nomination process, but all changes to already confirmed long term nominations must be rejected.

| RC | Reason Text | ITRs Action |
| --- | --- | --- |
| A57 | Deadline limit exceeded | Send last confirmed time series values |

### Intraday nomination process

After the publication of intraday auction results nominations of intraday rights can be sent or modified. During this period, the ITR may submit schedules. According to local market rules and the stage of the process, the TSO may send:

- confirmation that the schedule was received

- Positive or negative Acknowledgement Document

- Anomaly Report

In case of errors, the ITR should correct the nominations as soon as possible.

Nomination of capacity allocated in ID1:

Intraday nomination process respects the same rules as the Daily nomination process, but all changes to already confirmed long term and daily nominations must be rejected.

Nomination of capacity allocated in ID2:

Intraday nomination process respects the same rules as the Daily nomination process, but all changes to already confirmed long term, daily and ID1 nominations must be rejected. Nominations using transmission rights allocated in the ID2 auction must be submitted for the entire delivery day D period:

- for the first 12 hours of delivery day D (from 00:00 to 12:00), the nomination values shall be equal to zero,

- for the last 12 hours of delivery day D (from 12:00 to 24:00), non-zero values may be nominated.

For a daylight saving time transition day, the nomination shall contain 23/25 hours, with zero nomination values for hours 0–11 / 0–13.

| Use case | RC | Reason Text | BRPs Action |
| --- | --- | --- | --- |
| 1 | A57 | Deadline limit exceeded | Send last confirmed time series values |

## Matching process

Immediately after the nomination Gate Closure Time, the matching process begins. In this process, neighboring TSOs match the nominations of the parties and their counterparties that were submitted to both TSOs.

Preconditions:

Cross nominations (m:n), Business type A03 with CCT and CAI, InArea, OutArea, InParty, OutParty

Matching kind:

Double-sided matching with a correction cycles (depending on the process step). Rules according to /5/ UCTE SO-SO

Matching is performed on a single Time series basis, there is no aggregation of TSs.

Matching nominations shall have the same

- OutArea

- OutParty

- InArea

- InParty

- CCT

- CAI

This combination must be unique for one sender (see 3.3.2 ETSO ESS 3.3).

### Correction Cycle, Matching at COT

Correction cycle starts with the first matching.

As a result of each matching process, ITRs will be informed about confirmations and errors in nomination schedules via CNF and ANO-reports.

During the correction cycle, ITRs may send corrected nominations. Only mismatched Time series or Time series with exceeded rights can be renominated at this stage of the process. Already matched Time series must not be changed.

A CNF-report confirming all ITR nominations is sent only once and will not be repeated after subsequent matching processes during the correction cycle.

During the correction cycle, a new matching process between the TSOs will start every 15 minutes until cut off time (COT). If the matching procedure exceeds 15 minutes, the next matching cycle is skipped. When COT is reached, the final matching cycle starts.

If the matching is successful before COT, no additional matching cycle will be performed.

The correction cycle ends at the COT.

If the nominations do not match at COT, they are modified according to the following principles and in the following order:

- In case of a mismatch, the values are adjusted to the lower of the two values

- If there are no more mismatches and a PTR is still exceeded, the relevant nominations are curtailed pro rata. Values with decimals are rounded down to the next lower integer.

## Curtailment of nominations

In rare cases of emergency situation or Force Majeure, it may be necessary for TSOs to curtail matched nominations at the border. In such cases, as soon as the respective TSOs perform the curtailment of the concerned nominations, the relevant ITRs will be informed immediately.

Curtailment process:

- The respective TSOs calculate the curtailed nominations and round the curtailed values down to the nearest integer

- An extraordinary matching of the curtailed nominations is performed

- updated Confirmation Reports are sent to ITRs

Formula:

Curtailed nominations = matched nominations * reduction factor

The reduction factor is specified separately for each direction and for each hour of the respective day when curtailment is applied.

The reduction factor represents the percentage of Already Allocated Capacity (AAC) remaining after the reduction is applied.

For example, if capacity is reduced by 40%, the reduction factor = 0,6.

Curtailment is performed simultaneously by both TSOs. After a successful matching procedure, a new CNF document shall be sent to all ITRs. This CNF will contain:

- A70 Curtailment reason code in the header

- for all affected Time series: A86 Confirmation with adjustment at the Time series level.

# Appendix

## ETSO ESS 2.3/ESS 3.3

### Definitions

DtdVersion=2

DtdRelease=3

or

DtdVersion=3

DtdRelease=x

(according to local market rules)

Message Level

- MessageIdentification: provided by sender, unique per day

- MessageVersion: new content means new version

- MessageType: A01

- ProcessType: A01

- ScheduleClassificationType: A01

- Sender Identification: Party Code of ITR

- Sender Role: A01 (ITR)

- Receiver Identification: Party code of TSO

- Receiver Role: A04 (TSO)

- MessageDateTime: Current time

- ScheduleTimeInterval: Calendar day

Timeseries Level

- SendersTimeSeriesIdentification: provided by sender, constant per message and header of timeseries, unique in the message, and all following versions with the same message id and the same day.

- SendersTimeSeriesVersion: 
versioning according to local market rules

- BusinessType: A03

- Product: 8716867000016

- ObjectAggregation: A03

- In Area: The (control)area where the energy will be delivered

- Out Area: The (control)area where the energy is coming from

- InParty: ITR responsible for the nomination for the InArea

- OutParty: ITR responsible for the nomination for the OutArea

- CapacityContractType: A04 = yearly, A03 = monthly, A01= Daily, A07 = intraday

- CapacityAgreementIdentification: <retrieved from AO>

- MeasurementUnit: MAW

Period + Interval Level

- TimeInterval: equal to Scheduletimeinterval

- According to market rules in area

- Pos: According to market rules in area

- Qty: value in MW, 3 decimals can be used depending on local market rules, all decimals have to be in accordance with the current auction rules

Reason Level: not used

### Example

X and y for DtdRelease and DtdVersion according to local market rules

<?xml version="1.0" encoding="UTF-8"?>

<?xml-stylesheet type="text/xsl" href="schedule-document.xsl"?>

<ScheduleMessage DtdRelease="x" DtdVersion="y">

<MessageIdentification v="C_20071119"/>

<MessageVersion v="1"/>

<MessageType v="A01"/>

<ProcessType v="A01"/>

<ScheduleClassificationType v="A01"/>

<SenderIdentification codingScheme="A01" v="<EIC of ITR>"/>

<SenderRole v="A01"/>

<ReceiverIdentification codingScheme="A01" v="10XSK-SEPS-GRIDB"/>

<ReceiverRole v="A04"/>

<MessageDateTime v="2024-01-19T10:22:25Z"/>

<ScheduleTimeInterval v="2024-01-18T23:00Z/2024-01-19T23:00Z"/>

<ScheduleTimeSeries>

<SendersTimeSeriesIdentification v="SCHEDULE_35240"/>

<SendersTimeSeriesVersion v="1"/>

<BusinessType v="A03"/>

<Product v="8716867000016"/>

<ObjectAggregation v="A03"/>

<InArea codingScheme="A01" v="10Y1001C--000182"/>

<OutArea codingScheme="A01" v="10YSK-SEPS-----K"/>

<InParty codingScheme="A01" v="<EIC of ITR>"/>

<OutParty codingScheme="A01" v="<EIC of ITR>"/>

<CapacityContractType v="A01"/>

<CapacityAgreementIdentification v="XXXXXXXX-SK-UA-XXXX"/>

<MeasurementUnit v="MAW"/>

<Period>

<TimeInterval v="2024-01-18T23:00Z/2024-01-19T23:00Z"/>

<Resolution v="PT15M"/>

<Interval>

<Pos v="1"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="2"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="3"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="4"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="5"/>

<Qty v="0.000"/>

</Interval>

...

...

<Interval>

<Pos v="93"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="94"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="95"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="96"/>

<Qty v="0.000"/>

</Interval>

</Period>

</ScheduleTimeSeries>

<ScheduleTimeSeries>

<SendersTimeSeriesIdentification v="SCHEDULE_35240"/>

<SendersTimeSeriesVersion v="1"/>

<BusinessType v="A03"/>

<Product v="8716867000016"/>

<ObjectAggregation v="A01"/>

<InArea codingScheme="A01" v="10YSK-SEPS-----K"/>

<OutArea codingScheme="A01" v="10Y1001C--000182"/>

<InParty codingScheme="A01" v="<EIC of ITR>"/>

<OutParty codingScheme="A01" v="<EIC of ITR>"/>

<CapacityContractType v="A01"/>

<CapacityAgreementIdentification v="XXXXXXXX-UA-SK-XXXX"/>

<MeasurementUnit v="MAW"/>

<Period>

<TimeInterval v="2024-01-18T23:00Z/2024-01-19T23:00Z"/>

<Resolution v="PT15M"/>

<Interval>

<Pos v="1"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="2"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="3"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="4"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="5"/>

<Qty v="0.000"/>

</Interval>

...

...

<Interval>

<Pos v="93"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="94"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="95"/>

<Qty v="0.000"/>

</Interval>

<Interval>

<Pos v="96"/>

<Qty v="0.000"/>

</Interval>

</Period>

</ScheduleTimeSeries>

</ScheduleMessage>

## Referenced documents

- ETSO ESS 2.3

ETSO Scheduling System (ESS) Implementation Guide 2.3

- ETSO ECAN 4.0

ETSO Capacity Allocation and Nomination System (ECAN) Implementation Guide 4.0

- ENTSO-E  Code list

ENTSO-E General Code List For Data Interchange

- ENTSO-E Acknowledgement Document (EAD) 5.0

Implementation guide for the ESS (Acknowledgement Document)

- ETSO ESS 3.3

ETSO Scheduling System (ESS) Implementation Guide 3.3